Here is the step-by-step master guide to build and wire your entire AgriBot AI circuit on the Tinkercad workspace stage from scratch using a 9V Battery, an Arduino Uno, and the L293D motor driver.

### 1. Power Rail & Common Ground Setup

Before wiring components, set up your power distribution lines on the breadboard:

- Arduino 5V Power Line:

  - Wire Arduino 5V $\rightarrow$ Breadboard Positive (+) Rail (Red line).

- Arduino Ground Line:

  - Wire Arduino GND $\rightarrow$ Breadboard Ground (-) Rail (Black line).

- 9V Battery Ground Connection:

  - Wire 9V Battery Negative (-) $\rightarrow$ Breadboard Ground (-) Rail (Creates Common Ground).

### 2. L293D Motor Driver Pin Connections

Place the L293D IC across the center divider of the breadboard (notch facing up):

#### Power & Ground Pins

- Pin 16 (VCC1 - Logic): Wire to Breadboard Positive (+) Rail (5V from Arduino).

- Pin 8 (VCC2 - Motor Power): Wire directly to 9V Battery Positive (+).

- Pins 4, 5, 12, 13 (GND): Wire all four pins to the Breadboard Ground (-) Rail.

#### Control Signals (To Arduino)

- Pin 1 (Enable 1,2): Wire to Arduino Pin 10 (Left Motor PWM Speed).

- Pin 2 (Input 1): Wire to Arduino Pin 9 (Left Motor Direction).

- Pin 7 (Input 2): Wire to Arduino Pin 8 (Left Motor Direction).

- Pin 10 (Input 3): Wire to Arduino Pin 7 (Right Motor Direction).

- Pin 15 (Input 4): Wire to Arduino Pin 6 (Right Motor Direction).

- Pin 9 (Enable 3,4): Wire to Arduino Pin 5 (Right Motor PWM Speed).

#### Motor Outputs

- Left DC Motor Pins: Wire to Pin 3 (Output 1) and Pin 6 (Output 2).

- Right DC Motor Pins: Wire to Pin 14 (Output 4) and Pin 11 (Output 3).

### 3. Sensors & Relay Module Connections

#### HC-SR04 Ultrasonic Sensor

- VCC: Wire to Arduino 5V (Red Rail).

- GND: Wire to Common Ground (Black Rail).

- TRIG: Wire to Arduino Pin 3.

- ECHO: Wire to Arduino Pin 2.

#### Soil Moisture Sensor

- VCC: Wire to Arduino 5V (Red Rail).

- GND: Wire to Common Ground (Black Rail).

- SIG / OUT: Wire to Arduino Analog Pin A0.

#### 5V Relay Module (Water Pump Simulation)

- VCC: Wire to Arduino 5V (Red Rail).

- GND: Wire to Common Ground (Black Rail).

- IN: Wire to Arduino Pin 4.

- COM Terminal: Wire to 9V Battery Positive (+).

- NO (Normally Open) Terminal: Wire to DC Water Pump Positive (+).

- DC Water Pump Negative (-): Wire to Common Ground (Black Rail).

### 4. Verification Checklist Before Clicking "Start Simulation"

- Common Ground Check: Are the Arduino GND, 9V Battery Negative, L293D Pins (4, 5, 12, 13), and sensor GNDs all linked together on the same black breadboard rail?

- Logic vs. Motor Power: Is Pin 16 getting 5V from Arduino while Pin 8 gets 9V directly from the battery?

- Serial Monitor Setup: Once you paste the code and click Start Simulation, open the Serial Monitor at the bottom to observe distance measurements and soil moisture readings in real time.