#include <WiFi.h>

#include <HardwareSerial.h>

// --- PIN DEFINITIONS (Matching Blueprint) ---

// L298N Motor Driver

#define ENA 13

#define IN1 12

#define IN2 14

#define IN3 27

#define IN4 26

#define ENB 25

// Sensors & Actuators

#define SOIL_PIN 34    // ADC1 Pin

#define TRIG_PIN 5     // Ultrasonic Trigger

#define ECHO_PIN 18    // Ultrasonic Echo

#define RELAY_PIN 19   // Water Pump Relay

// Hardware Serial 2 for GPS

HardwareSerial gpsSerial(2);

// --- GLOBAL VARIABLES ---

int soilMoistureValue = 0;

long duration = 0;

int distanceCm = 0;

void setup() {

Serial.begin(115200);

// Initialize GPS Serial (RX2: GPIO 16, TX2: GPIO 17)

gpsSerial.begin(9600, SERIAL_8N1, 16, 17);

// Motor Driver Pins

pinMode(ENA, OUTPUT);

pinMode(IN1, OUTPUT);

pinMode(IN2, OUTPUT);

pinMode(IN3, OUTPUT);

pinMode(IN4, OUTPUT);

pinMode(ENB, OUTPUT);

// Sensor & Relay Pins

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

pinMode(RELAY_PIN, OUTPUT);

// Default State: Pump OFF (Relay active LOW or HIGH depending on module)

digitalWrite(RELAY_PIN, LOW);

Serial.println("AgriBot AI Core Firmware Initialized.");

}

void loop() {

// 1. Read Soil Moisture

soilMoistureValue = analogRead(SOIL_PIN);

// 2. Read Obstacle Distance (Ultrasonic)

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

duration = pulseIn(ECHO_PIN, HIGH);

distanceCm = duration * 0.034 / 2;

// 3. Safety Check: Emergency Stop if obstacle < 15cm

if (distanceCm > 0 && distanceCm < 15) {

stopMotors();

Serial.println("OBSTACLE DETECTED! Emergency Stop Active.");

}

// 4. Read GPS Telemetry Pass-through

while (gpsSerial.available() > 0) {

char c = gpsSerial.read();

// Raw NMEA data can be forwarded to your dashboard/serial monitor

}

delay(500);

}

// --- MOTOR CONTROL FUNCTIONS ---

void moveForward(int speed) {

analogWrite(ENA, speed);

analogWrite(ENB, speed);

digitalWrite(IN1, HIGH);

digitalWrite(IN2, LOW);

digitalWrite(IN3, HIGH);

digitalWrite(IN4, LOW);

}

void stopMotors() {

analogWrite(ENA, 0);

analogWrite(ENB, 0);

digitalWrite(IN1, LOW);

digitalWrite(IN2, LOW);

digitalWrite(IN3, LOW);

digitalWrite(IN4, LOW);

}

// --- IRRIGATION ACTION FUNCTION ---

void triggerIrrigation(int durationMs) {

Serial.println("Action Triggered: Irrigation Pump ON");

digitalWrite(RELAY_PIN, HIGH);

delay(durationMs);

digitalWrite(RELAY_PIN, LOW);

Serial.println("Action Completed: Irrigation Pump OFF");

}

### Component Diagnostics Checklist

Before running autonomous missions, verify hardware step-by-step:

- Power Verification: Adjust the LM2596 buck converter using a multimeter to confirm output is exactly 5.0V DC before connecting it to the ESP32 VIN pin.

- Motor Direction Test: Upload the sketch and call moveForward(150);. Verify all four wheels rotate in the correct direction (invert IN1/IN2 or IN3/IN4 wires if wheels run backward).

- Sensor Calibration: Test the soil moisture sensor in dry soil vs. water to log minimum and maximum ADC values on GPIO 34 for percentage mapping.